2321 Yellow Banks Road 
North Wilkesboro, North Carolina 28659
www.greenemountain.net

ENTER GM10 COUPON CODE ON OUR WEBSITE FOR A 10% DISCOUNT ON YOUR NEXT ORDER

Polaris RZR 4 accessories are available in Black, Blue and Black, Mossy Oak Break Up, Orange and Black, Red and Black, 1000D ACU Digital Military Spec fabric.

- Full Cab Enclosure with 4 removable doors. New mounting system helps ease initial install.

- Roof cover uses clamps instead of straps to mount to roll bar.

- Rear Windjammer wraps around roll bar back to itself for clean exterior look.

- Side bar pads, rear grab handle pad and tilt steering pad.

- Side enclosures work with our soft tops, windshields and rear windjammers.

Greene Mountain Outdoors

## Polaris RZR  and RZR4

POLARIS RZR 4 CAB ENCLOSURE

Installation Instructions

- From front of RZR 4®, install cover over roll cage/frame so straps with clips will engage plastic under front fenders.  Cover will pull over roll cage and fasten in the rear using clips at the front of the bed. Removal of rear seats may make this easier. Cab should be level and straight with cage frame.

- Zip Doors in place. (2 left, 2 right)

- Attach all clips around front fender openings and rear fender openings. Straps on center roll cage should be fastened once all clips are attached and cab looks level and tight.

- Before supplied Velcro can be installed, you must clean areas with alcohol.

- After good initial fit is achieved, install supplied Velcro around sides and door openings corresponding to Velcro mate on cover if desired.

Doors can be rolled up and stowed by unzipping front and rear zippers to the top of the door leaving zipper attached across the top of the door. Pull straps under rolled door and fasten to buckles sewn into outside of cab.

**Do Not Exceed 55 mph**

**Remove enclosures completely prior to transporting on highways, where speeds may exceed 55 mph

**Your new cab’s windows and doors are made of highest quality clear plastic.  It will however scratch, if not handled correctly

**Use mild soap and water only to clean all surfaces

Your new cover is guaranteed against any defects in material and workmanship for 90 days from the date of purchase.  Should any problems arise, please contact your dealer.  No warranty card is necessary.

WARNING:  Prolonged exposure to direct sunlight may result in fading and/or tendering of the material.  Do not store your ATV with fender cover installed in direct sunlight.  Damage resulting from prolonged exposure to direct sunlight cannot be warranted.

Your RZR 4® has several warning and caution labels around the machine that may be covered when this cover is installed. It is your responsibility as the owner to read and understand these warnings as well as making sure anyone who operates the UTV is aware of the proper operation.